
GLUTARALDEHYDE / FORMALDEHYDE LRPRIVATE 

réf : GLK100   Kit Complet Gutaraldehyde (ce kit fonctionne bien mais le changement de couleur n'est pas totalement netPrix = 92 Euros HT
Range
         0 - 25 ppm as formaldehyde          5 - 50 ppm as glutaraldehyde

Procedure
1.
Using the syringe provided take 80 ml of sample and transfer to the test jar.

2.
If the sample is known to contain the aldehyde alone then proceed to step 3.  If acidic or alkaline constituents may be present, the sample will require neutralisation to the thymolphthalein or phenolphthalein end point depending in test.  See Notes 
at the end of this test method. 

3.
Add one LEVEL red scoop of GL5 Powder  to the test jar and swirl gently until 
ALL of the powder has dissolved.


(IMPORTANT : ADD A LEVEL SCOOP NOT A HEAPED ONE).

4.
Add two drops of TY1 if testing for formaldehyde or PHENOLPHTALEIN if testing for glutaraldehyde per 10 ml of sample.  Swirl the jar gently to mix.  The sample will turn blue (TY1) or pink (PHENOLPHTALEIN)  if aldehyde is present.


(Omit this step if the sample has already been neutralised in step 2).

5.
Add reagent GL6 one drop at a time, mixing between each addition.  Count the  
number of drops of GL6 required for the colour to disappear.


formaldehyde ppm (mg/l)   =  No Drops GL6 used

Glutaraldehyde ppm (mg/l)  =  (No drops GL6 used - 2)  x  2

Notes

1.
Neutralisation Procedure :-

To the 80 ml of sample taken in step 1, add 2 drops of TY1 or PHENOLPHTALEIN depending on test  per 10 ml of sample and swirl to mix.  Note the colour produced.

a.
If a blue or pink solution is produced, add drops of GL6 solution until the colour JUST disappears.  DO NOT add any more drops than is necessary to decolourise the solution.


Proceed from step 3 above, omitting step 4.

b.
If a clear solution remains, add drops of 0.067N Sodium hydroxide (NaOH) solution 
until a blue or pink colour depending on test, JUST appears.  DO NOT add any 
more drops than is necessary to give the coloured solution.


Proceed from step 3, omitting step 4.

2.
When adding reagent GL6, hold the dropper bottle vertically upside down and allow the drops to form slowly and drop off under their own weight.  DO NOT SHAKE DROPS OFF.










page 2
